# SkyScouter Flight Control — The Plan, in Three Phases

Last updated: 2026-05-28 · branch feature/jetson-live-camera-runtime

This is the shared plan for turning SkyScouter from “a camera that locks onto a drone” into “an aircraft that flies itself at the drone.” We’re doing it in three steps so that each one can be flown and trusted before we add the next.

If you read nothing else, read this:

| Phase | What it does | Owner / status |
| --- | --- | --- |
| Phase 1 | Turn to face the target (yaw + hold altitude) | DONE & flown ✓ |
| Phase 2 | Match its height (climb / descend) | Colleague — next |
| Phase 3 | Close the distance (fly forward) | Ali — after |

Each phase is a small change in one file (skyscouter/flight/mavlink_link.py) plus a couple of config knobs. The vision side — detection, tracking, lock — is already done and doesn’t change. All three phases just consume the same per-frame GuidanceHint the pipeline already produces.

## How the whole thing fits together

The vision pipeline watches the camera, finds the drone, and every frame it hands out one small message — a GuidanceHint. That message says, in plain terms: “the target is this many degrees to the right and this many degrees up from the middle of the frame, and here’s how locked I am.” Our job on the flight side is to read that message and move the aircraft to shrink those numbers to zero.

The pipeline (everything up to the arrow is already built and doesn’t change):

camera  →  detector  →  tracker  →  lock FSM  →  GuidanceHint  →  MavlinkFlightLink  →  Pixhawk

(our three phases live here)

Each phase reads one part of that hint:

| Field in the GuidanceHint | What it means | Which phase uses it |
| --- | --- | --- |
| bearing_error_deg | target left / right of center | Phase 1 — yaw (turn to face) ✓ |
| elevation_error_deg | target above / below center | Phase 2 — climb (match height) |
| lock state = STRIKE_READY | locked, centered, stable | Phase 3 — forward velocity (close range) |

The three phases stack. Phase 2 keeps Phase 1’s yaw running and adds vertical motion. Phase 3 keeps both and adds forward motion, but only once the target is properly locked and centered.

How the aircraft is controlled. Everything goes over MAVLink to an ArduPilot autopilot (Pixhawk 6X) on a USB serial cable. We put the aircraft in GUIDED mode and stream it setpoints. We do not send land, return-to-home, or disarm from code — those stay on the pilot’s RC switch as the hardware safety net. That was true in the engineer’s old hand_track.py and it’s still true here.

## Phase 1 — Turn to face the target (DONE)

### What it does

The aircraft takes off to a fixed height, holds that height, and yaws (rotates its nose) to point at the locked drone. It doesn’t go up, down, forward, or sideways. It just turns to face. That’s the whole job of Phase 1, and it’s flying.

This is the same thing the engineer’s hand_track.py did on the bench (turn to face a hand), except the target now comes from the autonomous drone detector instead of a person waving, and there’s no mouse click anywhere.

### How it works under the hood

The flight link runs a background thread at a steady 30 Hz. When valid guidance starts arriving, it walks through this sequence once:

- Switch to GUIDED mode (DO_SET_MODE)

- Force-arm the motors (COMPONENT_ARM_DISARM, param2 = 2989.0 — the ArduPilot “skip pre-arm checks” magic number)

- Take off to the configured height (NAV_TAKEOFF, default 2 m)

- Settle — hold zero-yaw for a couple of seconds so it stabilizes

Then, every tick after that, it streams two messages:

- Hold altitude — SET_POSITION_TARGET_LOCAL_NED with type_mask 1531. That mask means “I’m only commanding a Z position; ignore all X/Y velocity.” In plain terms: stay at this height, don’t drift forward or sideways.

- Point at the target — CONDITION_YAW with a relative heading. We take the target’s left/right angle (filtered_bearing_error_deg), apply a 1° deadband so it doesn’t twitch when basically centered, and clamp it to ±15° per command so it turns smoothly instead of snapping.

If guidance goes stale (no valid hint for more than 0.5 s) or the lock is lost, it commands zero yaw and holds — it does not keep spinning toward a target it can no longer see.

### The seven MAVLink messages (this is the entire vocabulary)

| Message | When | Why |
| --- | --- | --- |
| DO_SET_MODE → GUIDED | once, at start | put the FC under our control |
| COMPONENT_ARM_DISARM (param2 = 2989.0) | once | force-arm the motors |
| NAV_TAKEOFF | once | climb to the fixed start height |
| SET_POSITION_TARGET_LOCAL_NED (mask 1531) | every tick | hold altitude, no XY motion |
| CONDITION_YAW (param4 = 1.0, relative) | every tick | turn the nose toward the target |
| HEARTBEAT | continuous | keep the link alive |
| REQUEST_DATA_STREAM | at start | ask the FC for telemetry back |

Those magic numbers (2989.0, 1531) came straight from the proven hand_track.py and are commented “do not change.” Phases 2 and 3 will relax the 1531 mask to allow vertical and forward motion — but only deliberately, and only in the file where it’s defined.

### The tests we ran (and what went wrong on the way)

Phase 1 wasn’t a clean first try, so here’s the honest record — it’s useful for Phases 2 and 3, because the same test discipline applies.

Test 1 — Sign check on the bench (dry-run, 765 frames). We ran the full pipeline against recorded footage with the flight link in dry-run (commands computed and logged, but the serial port never opened). We then read flight_commands.jsonl and confirmed: when the target is to the right of center, the commanded heading is positive; the deadband fired correctly when the target crossed the middle. Conclusion: the sign convention is correct, so we keep invert_yaw: false. No aircraft involved — this is how you prove direction safely before anything spins.

Test 2 — First live attempts FAILED (and why). On the first real option 6 runs the drone did nothing — only heartbeats went out, no arm, no takeoff. The autopilot was sitting in STABILIZE mode, and our sender thread had a logic bug: it checked “am I in GUIDED?” and bailed out before it ever sent the command to switch into GUIDED. Classic chicken-and-egg. Fixed by sending SET_MODE → GUIDED before that gate, and we added verbose [flight] logging that decodes every COMMAND_ACK to plain words (ACCEPTED / DENIED / etc.) so we can see exactly what the FC is doing.

Test 3 — Live flight SUCCESS. Next run: force-arm ACCEPTED, NAV_TAKEOFF to 2 m ACCEPTED, CONDITION_YAW ACCEPTED. The aircraft lifted off to 2 m, held, and yawed toward the target. The “unusual movements” we saw were exactly the intended yaw-to-target. Pilot landed it on the RC switch. That’s a working Phase 1.

The takeaway for everyone: dry-run first, read the JSONL, then fly. The verbose [flight] log telling you ACCEPTED vs DENIED is your best friend.

### Where Phase 1 lives

- Code: skyscouter/flight/mavlink_link.py (the MavlinkFlightLink class)

- Wiring: skyscouter/flight/factory.py (builds it from config, lazy-imports pymavlink)

- Config: the flight_control: block in configs/deploy_jetson_yolov26_lrdd_v2_siyi_a8_mini_1080p.yaml

- Log: flight_commands.jsonl (one row per decision, dry-run or live flagged)

## Phase 2 — Match the target’s height (colleague owns this)

### The goal

Right now the aircraft holds a fixed altitude. Phase 2 makes it climb or descend to line up vertically with the drone — the same idea as Phase 1’s yaw, but for up/down instead of left/right. When you’re done, the aircraft should keep the target centered both horizontally (Phase 1) and vertically (Phase 2).

### The good news: the vision side is already done

You do not need to touch detection, tracking, or guidance. The pipeline already computes the vertical angle for you and puts it in every GuidanceHint:

- elevation_error_deg — degrees the target is above (+) or below (−) center

- filtered_elevation_error_deg — the smoothed version (use this one, like Phase 1 uses filtered_bearing_error_deg)

- pitch_rate_cmd_deg_s — a pre-computed rate if you’d rather drive a rate

So Phase 2 is almost entirely a flight-side change. Your work happens in MavlinkFlightLink.

### What you actually change

Today the altitude-hold message is built in _send_altitude_hold_only():

target_ned_z_m = -float(self._guided_alt_m)   # fixed height, NED: down is +

That’s a constant. Phase 2 makes it move with the target. Two reasonable ways to do it (pick one, document which):

- Move the altitude setpoint (position). Keep the same 1531 mask, but ramp target_ned_z_m up or down based on filtered_elevation_error_deg. More conservative — you’re still commanding a position, just a changing one.

- Command a climb rate (velocity). Relax the 1531 type_mask to let the Z velocity (vz) term through, and set vz proportional to the elevation error, clamped to a safe rate. Smoother to control, but you’re changing the mask, so be deliberate about it.

Either way: add a deadband (don’t chase tiny errors), clamp hard (a max climb rate / max altitude band), and make both limits config values, not constants.

### New config knobs to add (suggested names)

pitch_follow_enabled: true     # master switch for Phase 2

max_climb_rate_m_s: 1.0        # clamp on vertical speed

alt_min_m: 1.5                 # never command below this

alt_max_m: 30.0                # never command above this

elevation_deadband_deg: 1.0    # ignore tiny vertical errors

### How “done” looks

- Bench dry-run shows sane vz/altitude in flight_commands.jsonl: target high in frame → command up; target low → command down; centered → no vertical command.

- Climb rate and altitude band are clamped and come from config.

- A guarded live test: the aircraft climbs/descends to keep the drone vertically centered while still yawing to face it.

### Test it the same way Phase 1 was tested

- Dry-run (option 5), read the JSONL, confirm vertical direction and clamps before anything flies. Vertical motion near the ground is less forgiving than yaw, so be extra strict here.

- Then a guarded live test with conservative clamps.

## Phase 3 — Close the distance (Ali owns this)

### The goal

With Phase 1 (facing) and Phase 2 (height) holding the target centered, Phase 3 adds forward velocity — the aircraft actually flies at the drone to close the range. This is the business end, so it has the strictest gate.

### The key rule: only when STRIKE_READY

Forward motion only happens when the lock FSM reports STRIKE_READY — meaning the target is locked, centered, big enough, and stable. In any other state (TRACKING, LOCKED-but-not-centered, LOST), forward velocity is zero. We don’t fly forward at something we’re not sure about.

### What you change

Forward motion means letting the body-frame X velocity (vx) through the setpoint. Today the 1531 mask explicitly forbids all X/Y velocity. Phase 3 relaxes that mask (when STRIKE_READY) so vx is allowed, and sets vx to a controlled closing speed.

- Build a STRIKE_READY-only path in the sender. Outside that state, vx = 0 and the aircraft behaves exactly like Phase 2.

- Closing speed should ramp, be clamped low at first, and be a config value.

- Keep yaw (Phase 1) and altitude/climb (Phase 2) running underneath — forward motion is added on top, not instead.

### New config knobs to add (suggested names)

forward_velocity_enabled: false   # master switch, OFF by default

max_forward_speed_m_s: 1.0        # start slow; clamp hard

strike_ready_only: true           # never move forward outside STRIKE_READY

max_close_range_time_s: 5.0       # safety: stop closing after N seconds

### How “done” looks

- vx is zero in every state except STRIKE_READY (prove this in dry-run JSONL).

- Closing speed is clamped and configurable, starts conservative.

- A guarded live test shows the aircraft only moves forward when fully locked and centered, and stops the moment lock is lost.

## Rules we all follow

These kept Phase 1 safe; they apply to all three phases.

- Dry-run before you fly. Every time. Run option 5, open flight_commands.jsonl, and confirm direction, signs, and clamps look right. Direction bugs are cheap to find on the bench and very expensive in the air.

- Edit only mavlink_link.py for flight behavior. The way data comes in is consume(hint). Everything else is config. You should almost never need to touch the vision pipeline.

- Tune through YAML, not code. Limits, deadbands, switches — all live in the flight_control: block of the deploy config. Hard-coded numbers are how you forget what a build was doing.

- Don’t change the magic numbers (2989.0, 1531) casually. Phase 2/3 will relax the 1531 mask on purpose — that’s fine — but do it in the one place it’s defined, with a comment saying why.

- The failsafe is “zero and hold.” Lost lock or stale guidance → stop commanding motion (zero yaw, hold altitude, zero forward). Land/RTL/disarm stay on the pilot’s RC switch.

- Geo-fence is ArduPilot’s job. We rely on the autopilot’s onboard geo-fence, not software in our pipeline.

- Clamp first, tune later. Always ship a new phase with conservative limits. It’s easy to loosen a clamp; it’s hard to un-crash a drone.

## How to run and test

Everything launches from jetson.sh:

| What | Command | Notes |
| --- | --- | --- |
| FC dry-run (safe) | ./jetson.sh run 5 | Full pipeline + flight logic; commands computed and logged but never sent. This is where you check signs and clamps. |
| FC LIVE | ./jetson.sh run 6 | Commands actually sent. Asks you to type fly to confirm; refuses to run without a terminal. |
| Live, higher takeoff | ./jetson.sh run 6 --fc-alt-m 5 | Sets a 5 m takeoff/hold height. |
| See the video while running | add --operator-view-mode mjpeg | View over WiFi in a browser; raw + annotated video are always saved anyway. |
| Headless field autostart | ./jetson.sh autostart (option 14) | Installs a systemd user service so it runs on boot with no display. Use SKYSCOUTER_MODEL=… to skip the model picker. |

One-time setup (already done on the field Jetson, noted here for a fresh box):

pip install pymavlink pyserial        # not in requirements-jetson.txt by design

sudo usermod -aG dialout $USER        # serial port access; re-login after

### Where things are

| Thing | Path |
| --- | --- |
| The flight link (edit here) | skyscouter/flight/mavlink_link.py |
| Builds it from config | skyscouter/flight/factory.py |
| Config block to tune | flight_control: in configs/deploy_jetson_yolov26_lrdd_v2_siyi_a8_mini_1080p.yaml |
| The contract from vision | GuidanceHint in skyscouter/schemas.py |
| Per-flight decision log | flight_commands.jsonl (in the run’s output folder) |
| Old reference code | ~/Documents/drone-control-jetson-main/hand_track.py (reference only — new work lands in SkyScouter) |

## Housekeeping note

The docstring at the top of mavlink_link.py still labels the later stages “Phase 4 / Phase 5.” That’s leftover numbering from an earlier draft — it should read Phase 2 (yaw + climb) and Phase 3 (yaw + climb + forward velocity) to match this plan. Whoever touches that file first, please fix the header so we’re all using the same numbers.