SKYSCOUTER

Product Requirements Document

Phase 1 - Radar-Cued Onboard Interceptor Perception, Reacquisition, Lock, and Strike-Ready Handoff

Counter-UAS Machine Vision / Onboard Edge Perception / MAVLink Integration

| Field | Value |
| --- | --- |
| Document version | v3.1 - Reality-aligned draft for leadership and engineering review |
| Document owner | Machine Vision Product / Engineering |
| Phase covered | Phase 1 only - onboard perception after radar cue handoff |
| Primary cue producer | External radar partner (ART / DARIT candidate; final partner TBD) |
| Onboard guidance consumer | Parham - Flight Controller / Guidance |
| Machine vision owners | Ali + Erfan |
| Human-in-the-loop authority | Mohammad - operator confirmation prior to launch / test execution |
| Demo intent | Full live: radar partner present, live cue, live interceptor flight, live target engagement test |
| Known platform facts | Approx. 60 km/h speed, approx. 15 min endurance, MAVLink command path, fixed-camera preferred |

Core scope correction from v2 to v3.1 Phase 1 no longer starts at ground-station drone detection. The radar partner is expected to provide the initial detection / cue. Skyscouter Phase 1 starts when the interceptor receives an approximate cue and must perform onboard search, reacquisition, visual lock, track continuity, and strike-ready handoff to Parham's flight controller.

Prepared as a working PRD, not a marketing document. Unknowns are explicitly preserved as integration risks.

# Document Control

| Version | Date | Change Summary |
| --- | --- | --- |
| v2.0 | 2026-04-27 | Ground-station target acquisition and cue handoff PRD. Useful structure but wrong Phase 1 ownership boundary after radar-partner decision. |
| v3.0 | 2026-04-27 | Initial reframe to radar-cued onboard interceptor perception. |
| v3.1 | 2026-04-27 | Leadership review pass: sharpened scope, fixed-camera baseline, low-cost sensor strategy, partner-interface assumptions, live-demo plan, updated requirements, risk register, validation gates, and open-question log. |

| Source / Input | How It Was Used |
| --- | --- |
| Skyscouter Machine Vision Team Onboarding v2 | Primary internal source: AI-Scout onboard vision responsibilities, camera-to-detection-to-guidance chain, <100 ms latency, Jetson/edge compute, EO/IR context, and Parham integration. |
| PRD v2.0 - Ground Station Target Acquisition | Retained useful discipline: target-state schema, track lifecycle, uncertainty handling, failure modes, bench/outdoor tests, TRL evidence mapping. Removed ground-station perception as Phase 1 baseline. |
| Ali answers to Claude question set | Authoritative reality update: radar partner will provide initial cue; drone hardware exists; camera not finalized; fixed camera preferred; MAVLink; approx. 60 km/h speed; 15 min endurance; human-in-loop launch by Mohammad; strike once locked; full live demo target. |
| Public information on ART and DARIT | Used only to shape partner-interface assumptions, not to assert unconfirmed technical performance. Both remain candidates until data-interface and accuracy details are confirmed. |

Leadership-level decision The right Phase 1 is not “build our own ground detector from scratch.” The right Phase 1 is “make the interceptor useful after a third-party radar cue.” This makes the program more credible, reduces duplicate work, and aligns Skyscouter effort with the part it must own: low-cost autonomous onboard target reacquisition and terminal lock.

# 1. Executive Summary

Skyscouter is building a low-cost hard-kill counter-UAS interceptor. The system concept is a modular engagement chain: an external radar partner performs initial surveillance and target detection; a human operator authorizes launch; the Skyscouter interceptor flies toward the radar cue; onboard perception reacquires the target, locks it, maintains track through closing geometry, and hands a guidance-valid target state to the flight controller.

This v3.1 PRD covers only Phase 1 of that chain: onboard interceptor perception from radar cue ingestion to visual lock and strike-ready handoff. It does not ask Ali and Erfan to develop the whole ground radar/detection stack. It assumes the initial cue is produced by a partner system, and it defines the contracts, algorithms, test gates, and acceptance criteria required for Skyscouter to turn that cue into a live onboard lock.

The product is not a generic YOLO demo. It is an onboard acquisition subsystem operating on a moving, vibrating, low-cost kamikaze platform with tight SWaP, limited compute, high relative closing speed, and minimal time to recover if the target is lost.

Phase 1 Definition of Done Given a live radar cue from the partner system and operator authorization from Mohammad, the interceptor launches or is positioned into an equivalent live-flight test profile, receives cue information over the agreed interface, performs onboard search and visual reacquisition using a fixed onboard EO camera baseline, promotes the target to LOCKED state under explicit confidence/stability gates, and publishes a guidance-valid target state to Parham's flight controller over MAVLink or the agreed internal interface while meeting latency, track-stability, and loss-of-track safety requirements.

# 2. Scope Guardrail

| In Scope for Phase 1 | Out of Scope for Phase 1 |
| --- | --- |
| Cue ingestion from radar partner or radar-cue simulator using the same schema | Building a Skyscouter-owned ground radar detector from scratch |
| Onboard EO camera selection, calibration, mounting strategy, and image-stability requirements | Full production sensor pod, rugged enclosure, or certified avionics package |
| Onboard target search, reacquisition, tracking, lock promotion, and loss-of-lock behavior | Multi-interceptor coordination, multi-site C2, or full military C2 integration |
| MAVLink / internal guidance handoff to Parham | Final rules of engagement policy or legal authorization framework |
| Bench, tethered, free-flight, and full-live demo validation path | All-weather operation, GPS-denied autonomy, or adversarial camouflage hardening |
| Operator confirm before launch / test run; autonomous terminal behavior after verified lock as a program assumption | Automated launch without human confirmation |

# 3. Product Problem

The interceptor cannot be expected to search the entire defended volume on its own. The radar partner provides the first cue, but that cue will arrive with uncertainty: range/bearing error, update latency, possible target maneuver after cue, coordinate-frame ambiguity, and possible class ambiguity. Once the interceptor is airborne, its onboard camera has a limited field of view and the target may initially be outside the frame. The onboard perception system must therefore search an uncertainty volume, reacquire a small moving target, establish a stable lock, and continuously publish a target state that the flight controller can trust.

The hard part is not initial detection in a clean recorded clip. The hard part is maintaining a reliable target state on a moving platform while both vehicles move, the target grows rapidly in frame, vibration and motion blur corrupt images, and the time available for recovery after a lost track collapses to seconds.

# 4. Concept of Engagement

| Stage | Owner | Product Expectation | Phase 1 Status |
| --- | --- | --- | --- |
| 1. Wide-area surveillance | Radar partner | Detect and track candidate UAS in live environment | External dependency |
| 2. Radar cue publication | Radar partner + Ali/Erfan interface adapter | Provide target state: position/bearing/range, timestamp, confidence, and uncertainty when available | In scope as interface contract |
| 3. Human confirmation | Mohammad | Confirm launch/test execution from live radar target context | In scope as gate; UI may be partner-provided or minimal Skyscouter tool |
| 4. Interceptor launch / mid-course | Parham + flight controller | Fly toward the cue / search volume; accept optional mid-course updates if available | In scope only at interface boundary |
| 5. Onboard search | Ali + Erfan perception | Use fixed onboard camera and cue geometry to scan/search the expected target volume | In scope |
| 6. Reacquisition | Ali + Erfan perception | Detect candidate drone in onboard video and start a track | In scope |
| 7. Lock | Ali + Erfan + Parham contract | Promote target to LOCKED only after confidence, stability, and geometry gates pass | In scope |
| 8. Terminal pursuit / impact | Parham flight controller | Use locked target state for terminal guidance | Interface in scope; detailed terminal guidance out of scope |

# 5. Confirmed Current Assumptions

| Area | Assumption | Implication |
| --- | --- | --- |
| Radar partner | ART and DARIT are candidate partners; detailed output format, accuracy, and update rate are not yet confirmed. | PRD must define a partner-agnostic cue adapter and a strict question list for partner meetings. |
| Demo | Radar partner will be physically present; demo target is full live, not recorded. | Validation must include live handoff and field rehearsal, not only replay. |
| Drone hardware | Drone airframe is mostly finalized; camera and compute are not finalized. | Sensor/compute decisions are still product decisions and must close early. |
| Camera mounting | Fixed camera is preferred; previous gimbal tests had jitter/shakes/distorted images. | Baseline shall avoid mechanical gimbal and use fixed optics plus software search/ROI behavior. |
| Compute | Current R&D options: Jetson Orin Nano, Orange Pi 5 Plus, RDK X5. | Reference platform should be selected by measured latency, power, and integration effort, not availability alone. |
| Command/data path | MAVLink is available. | Target-state handoff should either map to MAVLink or use a local onboard bridge to MAVLink. |
| Performance | Approx. interceptor speed 60 km/h; endurance approx. 15 min. | Search/reacquisition must happen quickly; test windows are manageable. |
| Autonomy | Drone is autonomous but can receive mid-flight commands. | System can accept optional cue updates but cannot depend on them in Phase 1. |
| Authority | Mohammad is human-in-loop before launch/test action. | PRD should require pre-launch human confirmation. |
| Lost target | Search pattern can be implemented. | Loss-of-lock behavior must be explicit and testable. |
| Strike policy | Assumption: once locked, strike behavior is automatic. | PRD should define LOCKED and STRIKE_READY gates; legal/ROE policy remains separate. |

# 6. Radar Partner Readiness Assessment

Public information indicates that both candidate partner directions are plausible for a modular C-UAS cue source, but neither can be treated as integration-ready until they provide a concrete output schema, coordinate frame, timestamping behavior, accuracy claims, update rate, and simulator or live feed for bench testing.

| Candidate | Publicly visible positioning | PRD implication |
| --- | --- | --- |
| Advanced Radar Technologies (ART) | Positions itself around C-UAS/UTM, 3D radars, software modules, high-resolution/high-refresh radars, and integrated surveillance systems. | Good candidate for radar cue producer. Need actual product/datasheet, output API, track format, update rate, coordinate frame, and C-UAS small-drone performance. |
| DARIT Technologies | IDS described as multimodal C-UAS/perimeter tracking using combinations of radar, vision, LiDAR, RF, and acoustic sensors, with fusion and visual classification; third-party reporting says DARIT won a CAD 1M IDEaS Urban Sandbox award with REMPART. | Strong ecosystem fit and likely easier Sandbox narrative. Need concrete live feed/API and whether DARIT provides radar itself or fuses partner radars. |

Partner interface stance The PRD shall not assume partner accuracy. Until measured, radar cue is treated as an uncertain search-volume initializer, not truth. Onboard perception must be able to recover from late, stale, wrong, or broad cues.

# 7. Recommended Phase 1 Architecture

The architecture is a cue-to-lock pipeline. It intentionally avoids a gimbal baseline and avoids relying on a ground-station detector owned by Skyscouter.

| Layer | Baseline | Rationale |
| --- | --- | --- |
| Radar cue adapter | Partner-agnostic adapter accepting ENU, geodetic, or bearing/range; converts to a common CUE_STATE object. | Keeps ART/DARIT interchangeable and lets Skyscouter test with a cue simulator before live partner feed. |
| Onboard camera | Fixed EO camera, low-latency, lightweight, calibrated. Optional dual fixed camera: wide acquisition + narrow lock. | Cheaper and mechanically simpler than gimbal; avoids jitter/latency from manual gimbal build. |
| Image stabilization | Mechanical damping + short exposure + software ROI/temporal filtering; no baseline mechanical gimbal. | Stabilizes enough for ML without adding cost/weight/control-loop delays. |
| Detector | Small YOLO-family / RT-DETR candidate benchmarked on target compute; production default chosen by recall-latency trade. | A smaller reliable model at high FPS beats a larger model that misses the latency budget. |
| Tracker | ByteTrack/SORT-class with cue-aware gating, plus lock state machine. | Tiny target crops make heavy re-ID less useful; state discipline matters more. |
| Search manager | Uses radar cue uncertainty + flight attitude to define image-space search ROI / scan behavior. | Prevents blind full-frame search when a rough cue exists. |
| Guidance handoff | MAVLink-native or onboard target-state bridge to Parham. | Matches current system and keeps Flight Controller ownership clean. |
| Operator gate | Human confirmation before launch/test; no continuous manual aiming. | Preserves test safety while keeping onboard autonomy meaningful. |

# 8. Camera and Mounting Strategy

The program goal is a low-cost kamikaze interceptor. That makes a high-quality gimbaled EO/IR turret a poor Phase 1 baseline unless it is already stable, cheap, and integrated. Current gimbal testing has shown jitter, shake, and distortion. The recommended Lockheed-style decision is therefore not “buy a better gimbal”; it is to eliminate the gimbal as a critical dependency for Phase 1.

## 8.1 Recommended Baseline

| Decision | Recommendation |
| --- | --- |
| Mount | Fixed mount, rigidly attached to airframe with vibration-isolation layer and known extrinsics. |
| Camera count | Minimum: one forward EO camera. Preferred: two fixed EO cameras - wide acquisition camera + narrower lock camera. |
| FOV | Wide acquisition: ~90-120 deg HFOV. Lock camera: ~45-70 deg HFOV. Avoid ultra-wide fisheye for final lock unless corrected. |
| Resolution / frame rate | 1080p60 minimum target; 4K useful if latency and compute allow. For terminal tracking, frame rate and exposure control can matter more than resolution. |
| Shutter/exposure | Global shutter preferred if affordable; rolling shutter acceptable only if vibration and exposure are controlled. Require manual exposure/short shutter. |
| IR | Do not make IR mandatory for Phase 1 low-cost baseline. Treat EO/IR as Phase 1B or Sandbox night extension if budget/weight allow. |
| Gimbal | Not baseline. Only reconsider if a commercial stabilized unit passes latency, jitter, cost, and weight gates. |

## 8.2 Why Fixed Camera is the Correct Phase 1 Bet

Lower cost and lower mechanical complexity match the kamikaze economics.

No gimbal control loop is required, eliminating a major source of delay and integration bugs.

Calibration is simpler and more stable when camera-to-body extrinsics are fixed.

Search can be handled by flight-body pointing, dual fixed FOV, and cue-aware ROI rather than moving optics.

The failure mode is easier to reason about: if the drone points at the search volume, the camera sees what the airframe sees.

Camera decision gate By Gate G1, test at least two low-cost EO options: one wide fixed camera and one narrower lock camera. Select by measured latency, blur under vibration, target pixels at 50-150 m, manual exposure support, weight, and integration effort. Do not select by resolution alone.

# 9. Compute Platform Recommendation

The compute decision should be made by flight-testable latency, power, thermal behavior, camera-driver maturity, TensorRT/model support, and MAVLink integration effort. Availability in the lab is not enough.

| Candidate | Role in Phase 1 | Decision logic |
| --- | --- | --- |
| Jetson Orin Nano | Recommended reference platform unless measured tests fail. | Best balance for CUDA/TensorRT/vision tooling. Likely most realistic for onboard ML development. |
| Orange Pi 5 Plus | Experimental / low-cost comparison. | May be attractive for BOM cost but carries higher ML deployment/integration risk; use only after benchmark evidence. |
| RDK X5 | Experimental / accelerator exploration. | Potentially efficient but must prove model conversion, camera ingest, and pipeline maturity. |
| Jetson AGX Orin | Development or premium fallback, not low-cost baseline. | Great for de-risking algorithms; may violate kamikaze cost/SWaP assumptions. |

# 10. Target State and Lock Lifecycle

The lock lifecycle is the product. It prevents a raw detection from becoming a dangerous guidance input.

| State | Meaning | Guidance Valid? | Exit / Transition |
| --- | --- | --- | --- |
| NO_CUE | No radar cue available or cue expired. | No | Wait for cue or operator-supplied test cue. |
| CUED | Radar cue received; interceptor has search volume but no onboard visual target. | No | Search manager starts ROI/scan behavior. |
| SEARCHING | Onboard perception actively searches expected image/space region. | No | Candidate detection appears or timeout triggers fallback. |
| ACQUIRED | Candidate target detected onboard but not yet stable. | No | Promote after N consistent detections and geometry sanity. |
| TRACKING | Persistent onboard track exists; confidence not yet high enough for terminal lock. | Limited / advisory | Promote when confidence, stability, and closure gates pass. |
| LOCKED | Track is stable, class is acceptable, and geometry is consistent with cue/flight path. | Yes | Publish guidance-valid target state to Parham. |
| STRIKE_READY | LOCKED plus terminal engagement constraints satisfied. | Yes, terminal | Final command policy owned by Parham/ROE; PRD defines only perception gate. |
| LOST | Track lost but recoverable within short time window. | No | Execute search pattern or abort based on timeout. |
| ABORTED | Search failed, cue stale, operator inhibit, or safety fault. | No | Flight controller executes safe behavior. |

# 11. Cue Schema - Partner Input

The radar partner may deliver geodetic position, local ENU, bearing/range, or a proprietary track object. Skyscouter should normalize all partner inputs into one internal CUE_STATE object.

| Field | Required? | Description |
| --- | --- | --- |
| cue_id | Yes | Partner or Skyscouter-generated unique cue ID. |
| timestamp_utc | Yes | Time of radar measurement, not time received. |
| source_partner | Yes | ART, DARIT, SIM, MANUAL_TEST, or other. |
| coordinate_type | Yes | GEODETIC, LOCAL_ENU, BEARING_RANGE, or PROPRIETARY_NORMALIZED. |
| position / bearing_range | Conditional | Target location in provided frame. |
| velocity | Preferred | Velocity vector or radial velocity if available. |
| uncertainty | Preferred; required before final live demo | Sigma/covariance or confidence/range gates. If absent, Skyscouter assigns conservative defaults. |
| class_label | Preferred | Drone / UAS / unknown; partner classification if available. |
| track_update_rate_hz | Metadata | Actual measured update cadence. |
| expiry_ms | Yes | After this age, cue cannot initialize guidance-valid lock. |

# 12. Onboard Target-State Output to Guidance

This message is produced by onboard perception and consumed by Parham. Exact transport may be MAVLink-native or a local process bridge, but the semantics must remain stable.

| Field | Type | Purpose |
| --- | --- | --- |
| schema_version | string | Versioned contract, e.g., skyscout.onboard_target.v1 |
| message_type | enum | TARGET_STATE, HEARTBEAT, NO_TARGET, SYSTEM_FAULT |
| timestamp_utc | string | Time of onboard observation / coordinate estimate |
| cue_id | string | Links onboard lock to radar cue |
| track_id | integer | Persistent onboard track ID |
| lock_state | enum | CUED, SEARCHING, ACQUIRED, TRACKING, LOCKED, STRIKE_READY, LOST, ABORTED |
| guidance_valid | boolean | True only for LOCKED/STRIKE_READY and no inhibit/fault |
| bbox_xywh | array | Image-space target box |
| line_of_sight_body | object | Azimuth/elevation or normalized ray in interceptor body frame |
| range_estimate_m | float/null | Optional onboard estimate; radar cue remains primary range source unless onboard geometry supports better estimate |
| confidence | float | Calibrated fused confidence |
| lock_quality | float | Composite stability score, separate from class confidence |
| latency_ms | float | Camera capture to publish latency |
| cue_age_ms | float | Age of latest cue used for search/lock association |
| sensor_sources | array | EO_WIDE, EO_LOCK, IR, SIM |
| model_version | string | Detector/tracker model package |
| fault_flags | array | VIBRATION_HIGH, BLUR_HIGH, CUE_STALE, CAMERA_DROP, MODEL_FAULT, etc. |

# 13. Functional Requirements

## 13.1 Cue Ingestion and Search Initialization

| ID | Requirement | Priority | Acceptance |
| --- | --- | --- | --- |
| FR-CUE-001 | System shall ingest a radar cue from a partner adapter or simulator using the same internal CUE_STATE schema. | P0 | Live or simulated cue initializes onboard SEARCHING state without code change. |
| FR-CUE-002 | System shall reject stale cues based on timestamp and configured expiry. | P0 | Expired cue cannot produce guidance_valid=true. |
| FR-CUE-003 | System shall support cue formats: LOCAL_ENU, GEODETIC, and BEARING_RANGE through normalization. | P1 | At least two formats demonstrated; third documented with test stub. |
| FR-CUE-004 | If partner uncertainty is missing, system shall assign conservative default search-volume uncertainty. | P0 | Missing sigma never treated as zero uncertainty. |
| FR-CUE-005 | System shall accept optional mid-flight cue updates but shall not depend on continuous radar updates for Phase 1. | P0 | Loss of updates after launch triggers autonomous onboard search/reacquisition behavior. |

## 13.2 Onboard Camera and Ingest

| ID | Requirement | Priority | Acceptance |
| --- | --- | --- | --- |
| FR-CAM-001 | System shall support at least one fixed onboard EO camera with known camera-to-body extrinsics. | P0 | Calibration package loaded and used in LOS output. |
| FR-CAM-002 | System shall measure camera-to-publish latency and dropped-frame rate continuously. | P0 | Latency and frame health visible in logs/HMI. |
| FR-CAM-003 | System shall detect excessive blur/vibration and set fault_flags accordingly. | P0 | Bench vibration test produces BLUR_HIGH or VIBRATION_HIGH flag. |
| FR-CAM-004 | System should support a dual fixed-camera mode: wide acquisition + narrower lock. | P1 | Pipeline can switch/associate tracks across two fixed EO streams. |
| FR-CAM-005 | System shall not require a mechanical gimbal for Phase 1 acceptance. | P0 | All core tests pass with fixed camera. |

## 13.3 Detection, Tracking, and Lock

| ID | Requirement | Priority | Acceptance |
| --- | --- | --- | --- |
| FR-DET-001 | System shall detect candidate drone targets in onboard video within the cue-informed search region. | P0 | Candidate detections appear in replay and live-flight tests. |
| FR-DET-002 | System shall output bbox, class/confidence, and model_version for each candidate target. | P0 | Fields present in target-state logs. |
| FR-TRK-001 | System shall maintain a persistent onboard track_id from ACQUIRED through LOCKED. | P0 | Track ID stability measured; ID switches reported. |
| FR-TRK-002 | System shall implement lock lifecycle states exactly as defined in Section 10. | P0 | State transitions validated in replay tests. |
| FR-LOCK-001 | System shall promote to LOCKED only after confidence, temporal stability, and cue-geometry sanity gates pass. | P0 | Single-frame false positives cannot become LOCKED. |
| FR-LOCK-002 | System shall compute lock_quality separate from class confidence. | P0 | Lock can be rejected for unstable track even with high detector confidence. |
| FR-LOST-001 | System shall execute configured loss-of-track behavior: short recovery search, then ABORTED or guidance-invalid state. | P0 | Target disappearance test produces LOST then recovery or ABORTED within threshold. |

## 13.4 Guidance and MAVLink Integration

| ID | Requirement | Priority | Acceptance |
| --- | --- | --- | --- |
| FR-IF-001 | System shall publish onboard target state to Parham's flight controller through MAVLink or a local bridge agreed by G1. | P0 | Parham test consumer parses all message types. |
| FR-IF-002 | System shall never publish guidance_valid=true outside LOCKED or STRIKE_READY. | P0 | Fault injection confirms no unsafe guidance-valid messages. |
| FR-IF-003 | System shall include cue_id and cue_age_ms in target-state messages. | P0 | Guidance can trace onboard lock back to radar cue. |
| FR-IF-004 | System shall support operator INHIBIT that forces guidance_valid=false. | P0 | INHIBIT test passes while detector sees a target. |
| FR-IF-005 | System shall publish HEARTBEAT independent of target availability. | P0 | Guidance can distinguish no target from perception failure. |

# 14. Non-Functional Requirements

| ID | Requirement | Priority | Acceptance |
| --- | --- | --- | --- |
| NFR-LAT-001 | P95 camera-capture-to-target-state-publish latency shall be <=100 ms on selected onboard compute. | P0 | Measured on flight-representative pipeline. |
| NFR-LAT-002 | P95 detector inference latency shall be <=50 ms target, <=70 ms max on selected model/input size. | P0 | TensorRT/accelerator benchmark report. |
| NFR-SWAP-001 | Camera + compute selection shall be justified by weight, power, thermal, cost, and integration complexity. | P0 | Trade study closed at G1/G2. |
| NFR-COST-001 | Baseline sensor stack shall preserve low-cost kamikaze economics; expensive EO/IR gimbal is non-baseline. | P0 | BOM estimate reviewed by leadership. |
| NFR-REL-001 | System shall fail guidance-invalid on stale cue, camera drop, model fault, excessive blur, or loss of lock. | P0 | Fault-injection test suite passes. |
| NFR-OBS-001 | Every run shall produce replayable logs: cue input, camera timestamps, detections, state transitions, target messages, and fault flags. | P0 | One recorded flight replayed deterministically. |
| NFR-MAINT-001 | Thresholds, camera calibration, cue-expiry, lock gates, and transport settings shall be externalized in config. | P0 | No code change needed for field tuning. |

# 15. Acceptance Criteria

| Category | Minimum Phase 1 | Target Phase 1 | Notes |
| --- | --- | --- | --- |
| Cue ingestion | Partner or simulator cue initializes onboard search. | Live partner cue initializes onboard search in field. | Simulator accepted for early gates only. |
| Onboard reacquisition | Target acquired from cue-informed search at representative range. | Target acquired within <=3 s after entering camera FOV. | Exact range determined by camera choice. |
| Lock promotion | LOCKED state requires >=N stable frames and geometry sanity. | False single-frame targets never promote to LOCKED. | N configurable; set by test. |
| Latency | P95 <=100 ms capture-to-publish. | P95 <=80 ms. | On selected onboard compute. |
| Loss recovery | LOST entered within threshold; recovery search starts automatically. | Recovery or ABORTED decision within <=2 s. | Supports search pattern. |
| MAVLink/guidance handoff | Parham consumer parses target-state, heartbeat, no-target, fault. | Bench + flight logs show expected FC behavior. | Transport frozen at G1. |
| Live demo | Full live chain rehearsed at least twice before Sandbox. | Three consecutive dry runs pass. | Radar partner physically present. |

# 16. Test Plan

## 16.1 Bench

Cue simulator sends LOCAL_ENU, GEODETIC, and BEARING_RANGE cases into adapter.

Replay onboard camera clips through detector/tracker/state machine.

Synthetic stale cue, wrong cue, delayed cue, and missing uncertainty tests.

MAVLink/bridge contract tests with Parham's consumer.

Vibration/blur bench rig test with fixed camera and representative motor/airframe vibration.

Fault injection: camera disconnect, model process kill, frame starvation, operator inhibit.

## 16.2 Tethered / Captive Flight

Interceptor powered and vibrating under flight-like conditions; camera stream and compute thermal behavior measured.

Target drone or moving visual target presented in known positions; state transitions verified.

Search behavior verified without committing to free-flight risk.

## 16.3 Free Flight

Interceptor receives cue and flies toward expected search region under operator-approved test conditions.

Onboard perception demonstrates ACQUIRED -> TRACKING -> LOCKED on live target.

Loss-of-track scenario tested with target crossing out of FOV or rapid maneuver.

MAVLink messages logged end-to-end with timestamps.

## 16.4 Full Live Integrated Demo Rehearsal

Radar partner present with live radar system.

Mohammad confirms launch/test action.

Skyscouter interceptor receives live cue, performs onboard search/reacquisition, achieves LOCKED, and publishes guidance-valid target state.

End-to-end log package produced immediately after run.

# 17. Milestones and Gates

| Gate | Target | Exit Criteria |
| --- | --- | --- |
| G0 - PRD / architecture freeze | Immediate | v3.1 scope accepted: radar partner owns initial detection; Skyscouter owns onboard reacquisition/lock. |
| G1 - Partner/cue contract + camera decision | +2 weeks | Partner question list answered enough to define adapter; fixed camera candidate selected; MAVLink/bridge path chosen. |
| G2 - Compute/model benchmark | +4 weeks | Detector/tracker runs on selected compute with measured latency; cost/SWaP trade closed. |
| G3 - Bench cue-to-lock replay | +6 weeks | Simulated cue -> onboard replay -> LOCKED -> guidance message passes. |
| G4 - Tethered/captive flight | +8 weeks | Camera vibration, blur, thermal, and latency proven under flight-like conditions. |
| G5 - Free-flight onboard lock | +10 weeks | Live target reacquired and locked in flight; Parham receives valid target state. |
| G6 - Partner integrated field test | +12 weeks | Live radar cue from partner drives onboard search and lock; logs reviewed. |
| G7 - Sandbox-ready rehearsal | Before shipment | Two or more consecutive full-live rehearsals pass with partner and operator procedure. |

# 18. Risk Register

| Risk | Impact | Mitigation | Owner |
| --- | --- | --- | --- |
| Radar partner output unknown / late | Cannot test live cue chain in time. | Build cue simulator now; require partner schema/API by G1; normalize all formats through CUE_STATE. | Ali + Erfan |
| Fixed camera cannot reacquire target from broad cue | Interceptor searches wrong volume or misses target. | Use dual fixed FOV if needed; constrain cue uncertainty; implement search pattern; require cue-quality gates. | Ali |
| Gimbal temptation returns late | Schedule slips due mechanical/control complexity. | Keep gimbal non-baseline; only accept if it passes a written trade and measured jitter/latency gate. | Leadership |
| Onboard compute too slow | Latency exceeds target; lock unstable. | Benchmark model sizes early; prefer Orin Nano reference; quantize/prune; reduce input size with cue-informed ROI. | Erfan |
| Motion blur/vibration breaks detection | Detector fails during real flight despite bench success. | Short exposure, rigid mount, damping, vibration bench test before free flight, blur flag in logs. | Hardware + Ali |
| MAVLink integration ambiguity | Perception and guidance disagree on frames/states. | Freeze schema and frame conventions at G1; build Parham test consumer. | Parham + Ali |
| False lock leads unsafe terminal behavior | System commits to bird/clutter/wrong object. | LOCKED requires temporal stability, cue consistency, operator inhibit, and fault-safe guidance_invalid behavior. | Ali + Erfan |
| Full live demo too ambitious | Component demos lose points; full live may fail. | Stage rehearsals: bench -> tethered -> free flight -> partner integrated; preserve fallback demo as contingency, not primary. | Program |

# 19. Open Questions for Partner and Internal Team

| Question | Owner | Needed By |
| --- | --- | --- |
| What exact radar cue output format will partner provide: lat/lon/alt, ENU, bearing/range, track object, or API stream? | Radar partner / Ali | G1 |
| Do cues include uncertainty/covariance/confidence? If not, what empirical accuracy can be measured? | Radar partner | G1 |
| Is radar update continuous after launch or initial cue only? What update rate? | Radar partner | G1 |
| What transport/interface: UDP, TCP, REST, serial, MAVLink, SDK, file stream? | Radar partner | G1 |
| Will partner provide a simulator/log replay feed before live field test? | Radar partner | G1 |
| Which fixed EO camera/lens is selected for baseline? Is dual-camera accepted? | Ali + Hardware | G1 |
| Which compute board becomes reference platform? | Ali + Erfan + Parham | G2 |
| Exact MAVLink message mapping or bridge format? | Parham | G1 |
| What is the safe behavior after LOST: search, abort, return, loiter, or continue mid-course? | Parham + Mohammad | G2 |
| What are the exact test constraints for live impact at Suffield? | Program leadership | G3 |

# 20. Recommended Immediate Next Actions

| Priority | Action | Why |
| --- | --- | --- |
| 1 | Send partner questionnaire to ART and DARIT immediately. | The PRD cannot commit interface/accuracy without their cue details. |
| 2 | Build a radar-cue simulator this week. | Allows Ali/Erfan/Parham to start integration before partner feed exists. |
| 3 | Freeze fixed-camera baseline and stop treating gimbal as core path. | Protects schedule and low-cost thesis. |
| 4 | Run a 2-day camera/compute bakeoff: Orin Nano + 1080p60 + candidate models. | Reality check for latency and image quality. |
| 5 | Create Parham target-state consumer stub. | Prevents late MAVLink/schema mismatch. |
| 6 | Define LOCKED / STRIKE_READY gates in writing with Mohammad and Parham. | Prevents unsafe or ambiguous terminal behavior during live test. |

# Appendix A - Partner Questionnaire

What exact output object do you provide per target track? Please include a sample message.

What coordinate frame is used? If geodetic, what altitude datum? If local, how is the origin defined?

Do you output covariance/sigma/confidence? If not, what is measured accuracy vs. Micro/Mini UAS at relevant ranges?

What is update rate and latency from radar measurement to external output?

Do you maintain a persistent target ID? How do you handle track loss, reacquisition, and ID switches?

Do you classify drone vs bird vs clutter? What confidence field is provided?

What interface is available for integration: UDP, TCP, REST, serial, MAVLink, SDK, C2 feed?

Can you provide a simulator or recorded track feed for bench integration before live field work?

Will your system be physically present and operated by your team during the full-live demo?

What environmental and deployment constraints do you require at the test site?

# Appendix B - Example CUE_STATE

{
  "schema_version": "skyscout.cue_state.v1",
  "cue_id": "partner_2026_09_14_000231",
  "timestamp_utc": "2026-09-14T18:42:11.120Z",
  "source_partner": "DARIT_OR_ART_TBD",
  "coordinate_type": "LOCAL_ENU",
  "target_position_enu_m": { "east": 182.4, "north": 431.2, "up": 74.0 },
  "target_velocity_enu_mps": { "east": -4.2, "north": 1.1, "up": 0.3 },
  "position_uncertainty_m": { "sigma_east": 25.0, "sigma_north": 25.0, "sigma_up": 15.0 },
  "class_label": "uas",
  "class_confidence": 0.78,
  "track_update_rate_hz": 5,
  "expiry_ms": 1500
}

# Appendix C - Example Onboard Target State

{
  "schema_version": "skyscout.onboard_target.v1",
  "message_type": "TARGET_STATE",
  "timestamp_utc": "2026-09-14T18:42:14.205Z",
  "cue_id": "partner_2026_09_14_000231",
  "track_id": 12,
  "lock_state": "LOCKED",
  "guidance_valid": true,
  "bbox_xywh": [812, 376, 28, 19],
  "line_of_sight_body": { "azimuth_deg": 4.8, "elevation_deg": -1.2 },
  "confidence": 0.84,
  "lock_quality": 0.79,
  "latency_ms": 68.4,
  "cue_age_ms": 935,
  "sensor_sources": ["EO_LOCK"],
  "model_version": "phase1_lock_yolo_trt_2026_07_20",
  "fault_flags": []
}

# Appendix D - Source Notes

Internal onboarding source states that AI-Scout vision must detect/classify/intercept unauthorized drones, output bboxes/class/range estimates, benchmark on edge compute, integrate EO/IR sensors, and validate with Parham's guidance module under latency constraints. This PRD preserves those engineering expectations but moves the Phase 1 starting point from ground-station detection to onboard perception after radar cue.

Public ART information positions ART around C-UAS/UTM, 3D radars, software modules, high-resolution/high-refresh radars, and integrated surveillance systems. Public DARIT information positions IDS/REMPART around multimodal C-UAS and perimeter tracking using radar, vision, LiDAR, RF, and acoustic fusion. These are treated as candidate-partner signals, not as confirmed integration specifications.

# Appendix E - Review Checklist

| Reviewer Lens | Questions to Ask Before Approval |
| --- | --- |
| CTO | Is the architecture testable before partner feed exists? Are unknowns isolated behind adapters? Is fixed-camera baseline technically defensible? |
| VP Product | Does Phase 1 solve the right product slice for Sandbox? Is the live demo credible? Does the narrative show ecosystem integration rather than duplicated radar work? |
| Flight Controls | Are target states, lock states, and guidance_valid semantics unambiguous? Are frames and timestamps sufficient? |
| Machine Vision | Are model, camera, tracking, and loss-of-lock requirements measurable and realistic on low-cost hardware? |
| Program / Safety | Does human confirmation exist before launch/test? Are false-lock and loss-of-lock failures handled safely? |